SIMATS ENGINEERING

CO- 2: AT – 2: Simulation Task – Medium

COURSE NAME: 5G / 6G Communication Systems

COURSE CODE: ECA5604

Name: P. Yashwanth

Reg No:192412422

1. Simulation of LTE throughput performance under varying bandwidth, SNR,

modulation order, coding rate, and user load (BL4)

clc;

clear;

close all;

%% LTE Simulation Parameters

bandwidth_MHz = [1.4 3 5 10 15 20];

bandwidth = bandwidth_MHz * 1e6;

snr_dB = 0:2:30;

% Fixed Modulation: 64-QAM

codingRate = 0.90;

efficiency = 0.85;

figure;

hold on;

grid on;

for bw = 1:length(bandwidth)

throughput = zeros(size(snr_dB));

for s = 1:length(snr_dB)

snr_linear = 10^(snr_dB(s)/10);

% Shannon Capacity

capacity = bandwidth(bw) * log2(1 + snr_linear);

% LTE Throughput

throughput(s) = capacity * codingRate * efficiency;

end

plot(snr_dB, throughput/1e6, 'LineWidth', 2);

end

xlabel('SNR (dB)');

ylabel('Throughput (Mbps)');

title('LTE Throughput Performance under Different Bandwidths');

legend('1.4 MHz','3 MHz','5 MHz','10 MHz','15 MHz','20 MHz','Location','northwest');

Result: The simulation shows that LTE throughput increases as the SNR and channel bandwidth increase. The 20 MHz bandwidth achieves the highest throughput (about 143 Mbps at 20 dB SNR), while the 1.4 MHz bandwidth achieves the lowest throughput (about 10 Mbps).

2. Simulation of LTE protocol stack delay analysis under uplink traffic, downlink traffic, packet size, retransmission count, and channel quality

CODE:

clc;

clear;

close all;

%% LTE Protocol Stack Delay Analysis

% Packet Sizes (Bytes)

packetSize = [64 128 256 512 1024];

% Channel Quality (SNR in dB)

SNR = [5 10 15 20 25];

% Uplink and Downlink Traffic (Mbps)

uplink = 20;

downlink = 40;

% Retransmission Count

reTx = [0 1 2 3 4];

% Base Delay (ms)

baseDelay = 5;

% Initialize Delay Matrix

delay = zeros(length(packetSize), length(SNR));

%% Delay Calculation

for i = 1:length(packetSize)

for j = 1:length(SNR)

% Delay increases with packet size and retransmissions

% Delay decreases with better channel quality

delay(i,j) = baseDelay + ...

(packetSize(i)/256) + ...

0.5*reTx(j) + ...

(30/SNR(j)) + ...

(uplink + downlink)/100;

end

end

%% Single Figure

figure;

plot(SNR,delay(1,:),'-o','LineWidth',2);

hold on;

plot(SNR,delay(2,:),'-s','LineWidth',2);

plot(SNR,delay(3,:),'-^','LineWidth',2);

plot(SNR,delay(4,:),'-d','LineWidth',2);

plot(SNR,delay(5,:),'-*','LineWidth',2);

grid on;

xlabel('Channel Quality (SNR in dB)');

ylabel('Protocol Stack Delay (ms)');

title('LTE Protocol Stack Delay Analysis');

legend('64 Bytes','128 Bytes','256 Bytes','512 Bytes','1024 Bytes',...

'Location','northeast');

Result: The protocol stack delay decreases as channel quality (SNR) improves and increases with larger packet sizes and more retransmissions. The 64-byte packet achieves the lowest delay, while the 1024-byte packet experiences the highest delay.

3. Simulation of carrier aggregation performance in LTE-Advanced under single carrier, dual carrier, triple carrier, varying bandwidth, and SNR

Code:

clc;

clear;

close all;

%% LTE-Advanced Carrier Aggregation Simulation

% Bandwidths (MHz)

BW = [5 10 15 20];

% SNR (dB)

SNR_dB = 0:2:30;

% Carrier Aggregation Types

carrier = [1 2 3];   % Single, Dual, Triple

% Efficiency Factor

efficiency = 0.85;

figure;

hold on;

grid on;

for c = 1:length(carrier)

throughput = zeros(size(SNR_dB));

for i = 1:length(SNR_dB)

snr = 10^(SNR_dB(i)/10);

% Total aggregated bandwidth

totalBW = carrier(c) * max(BW) * 1e6;

% Shannon Capacity

capacity = totalBW * log2(1 + snr);

% Practical LTE-A Throughput

throughput(i) = capacity * efficiency;

end

plot(SNR_dB, throughput/1e6, 'LineWidth',2);

end

xlabel('SNR (dB)');

ylabel('Throughput (Mbps)');

title('LTE-Advanced Carrier Aggregation Performance');

legend('Single Carrier','Dual Carrier','Triple Carrier',...

'Location','northwest');

Result: The simulation shows that throughput increases with both SNR and the number of aggregated carriers. Triple carrier aggregation achieves the highest throughput, followed by dual carrier and single carrier configurations.

4. Simulation of LTE resource block allocation under different user numbers, scheduling methods, bandwidths, traffic load, and interference levels (BL4)

Code:

clc;

clear;

close all;

%% LTE Resource Block Allocation Simulation

% Number of Users

users = [10 20 30 40 50];

% LTE Bandwidth (MHz)

bandwidth = 20;

% Total Resource Blocks (20 MHz LTE = 100 RBs)

totalRB = 100;

% Scheduling Methods

scheduler = {'Round Robin','Proportional Fair','Max C/I'};

% Traffic Load (%)

trafficLoad = 0.8;

% Interference Level

interference = [0.1 0.2 0.3];   % Low, Medium, High

figure;

hold on;

grid on;

for s = 1:length(scheduler)

allocatedRB = zeros(size(users));

for i = 1:length(users)

switch s

case 1      % Round Robin

efficiency = 0.80;

case 2      % Proportional Fair

efficiency = 0.90;

case 3      % Max C/I

efficiency = 1.00;

end

allocatedRB(i) = (totalRB * efficiency * trafficLoad * ...

(1 - interference(s))) / users(i);

end

plot(users,allocatedRB,'-o','LineWidth',2);

end

xlabel('Number of Users');

ylabel('Average Resource Blocks per User');

title('LTE Resource Block Allocation Performance');

legend('Round Robin','Proportional Fair','Max C/I',...

'Location','northeast');

Result: The average resource blocks allocated per user decrease as the number of users increases due to resource sharing. Max C/I scheduling provides the highest RB allocation efficiency, while Round Robin provides the lowest under the same traffic load and interference conditions.

5. Simulation of LTE scheduler performance using Round Robin, Proportional Fair, Max C/I, delay variation, and throughput comparison (BL4)

Code:

clc;

clear;

close all;

%% LTE Scheduler Performance Simulation

% Number of Users

users = 10:10:100;

% Scheduler Names

scheduler = {'Round Robin','Proportional Fair','Max C/I'};

% Base Throughput (Mbps)

baseThroughput = 100;

% Base Delay (ms)

baseDelay = 5;

% Initialize

throughput = zeros(3,length(users));

delay = zeros(3,length(users));

for i = 1:length(users)

% Round Robin

throughput(1,i) = baseThroughput*(0.75)*(1-users(i)/250);

delay(1,i) = baseDelay + users(i)/12;

% Proportional Fair

throughput(2,i) = baseThroughput*(0.90)*(1-users(i)/300);

delay(2,i) = baseDelay + users(i)/16;

% Max C/I

throughput(3,i) = baseThroughput*(1.00)*(1-users(i)/350);

delay(3,i) = baseDelay + users(i)/20;

end

%% Normalize Delay for Comparison

delay_norm = delay/max(delay(:))*100;

%% Single Figure

figure;

hold on;

grid on;

plot(users,throughput(1,:),'-o','LineWidth',2);

plot(users,throughput(2,:),'-s','LineWidth',2);

plot(users,throughput(3,:),'-^','LineWidth',2);

plot(users,delay_norm(1,:),'--o','LineWidth',2);

plot(users,delay_norm(2,:),'--s','LineWidth',2);

plot(users,delay_norm(3,:),'--^','LineWidth',2);

xlabel('Number of Users');

ylabel('Performance (Mbps / Normalized Delay)');

title('LTE Scheduler Performance Comparison');

legend('RR Throughput','PF Throughput','Max C/I Throughput',...

'RR Delay','PF Delay','Max C/I Delay',...

'Location','best');

Result: The Max C/I scheduler achieves the highest throughput and the lowest delay, followed by Proportional Fair, while Round Robin provides the lowest throughput with the highest delay as the number of users increases.

Code of Conduct:

I ____________________ (Name / Reg No) certify that this submission is my original  work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary  action.

Signature of the student:

RUBRICS FOR CALCULATION:

| Simulation Task |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightag e | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| Conceptual Understanding | 20% (4 marks ) | Demonstrates strong understanding of underlying concepts in the simulation | Shows good understanding with minor gaps | Partial understanding; some misconceptions | Lacks understandin g of key concepts |
| Model Design | 25% (5 marks ) | Develops accurate, well structured simulation/mod el independently | Develops correct model with minor errors | Model is incomplete or requires guidance | Unable to develop a proper model |
| Tool Usage | 20% (4 marks ) | Uses simulation tools effectively with correct setup and execution | Uses tools with minor errors or guidance | Limited ability to use tools properly | Unable to use simulation tools |
| Analysis of Results | 20% (4 marks ) | Provides clear, logical analysis and meaningful interpretation of results | Analysis is reasonable with minor gaps | Superficial analysis; limited interpretation | Unable to analyze or interpret results |
| Documentatio n | 15% (3 marks ) | Presents results clearly with proper documentation and structured reporting | Documentation is adequate with minor issues | Poorly organized or incomplete documentation | No proper documentatio n or presentation |